AAE4011 — Artificial Intelligence in Unmanned Autonomous Systems

Group Project Report

UAV Obstacle Avoidance Path Planning in Urban Low-Altitude Logistics Using Deep Reinforcement Learning

Group 12

| Name | Student ID | Role |
| --- | --- | --- |
| LIU Ziyang | 22100364D | Algorithm Engineer — PPO Implementation & Model Training |
| WANG Yetian | 22103334D | Simulation Engineer — AirSim Environment & Scene Design |
| CAI Jialiang | 22100304D | Project Lead — Overall Planning, Evaluation & Documentation |

Department of Aeronautical and Aviation Engineering

The Hong Kong Polytechnic University  |  Semester 2, 2025/2026

Supervisor: Dr. Weisong WEN

GitHub: https://github.com/Victor7890-svg/drone-ppo-navigation

# 1. Introduction

The rapid growth of urban air mobility has created significant demand for autonomous drone navigation systems capable of operating reliably in complex, obstacle-rich environments. Urban low-altitude logistics — the delivery of goods via unmanned aerial vehicles (UAVs) within city airspaces — is projected to become a multi-trillion yuan industry by 2030. However, realising this potential requires solving a fundamental challenge: how can a drone navigate autonomously through dense urban environments, avoiding static structures and dynamic obstacles in real time?

Traditional path planning algorithms such as A* and Rapidly-exploring Random Trees (RRT) have been widely applied to robotic navigation problems. While these methods can find geometrically valid paths, they suffer from high computational complexity and limited adaptability in dynamic environments. In urban settings characterised by irregular building layouts and variable obstacle density, these limitations become significant barriers to practical deployment.

This project addresses this challenge by applying Deep Reinforcement Learning (DRL), specifically the Proximal Policy Optimization (PPO) algorithm, to train a UAV agent to perform end-to-end autonomous navigation. Rather than relying on pre-computed maps or hand-crafted rules, the PPO agent learns from interaction with a simulated urban environment, developing a robust navigation policy capable of millisecond-level responses.

The simulation environment was built using Microsoft AirSim integrated with Unreal Engine 4.27, providing a physically realistic urban scene with multiple block obstacles. A curriculum learning strategy was employed to progressively increase task difficulty, enabling the agent to generalise from simple short-range navigation to a complex long-range path of approximately 138 metres. The trained model achieved a 100% success rate across 20 evaluation episodes with zero collisions, exceeding the original project target of 90%.

# 2. Problem Statement

The core problem addressed in this project is autonomous obstacle avoidance path planning for UAVs operating in urban low-altitude environments. Specifically, the agent must navigate from a fixed start position to a designated target location within a cluttered environment, while satisfying three constraints:

- Safety: the drone must not collide with any obstacle at any point during flight.

- Efficiency: the navigation path should be as direct as possible, minimising unnecessary detours and total flight time.

- Autonomy: the drone must make all navigation decisions independently, without human intervention or prior map knowledge.

The limitations of existing approaches motivate the use of deep reinforcement learning. A* and Dijkstra's algorithm require a complete, pre-computed map of the environment. RRT-based methods are probabilistically complete but computationally expensive in dense environments. Deep reinforcement learning, by contrast, learns a generalised reactive policy through interaction, enabling real-time decision-making without explicit map construction or runtime planning.

# 3. Methodology

## 3.1 Algorithm: Proximal Policy Optimization (PPO)

PPO was selected as the core learning algorithm due to its strong performance in control tasks, high sample efficiency, and training stability. Unlike earlier policy gradient methods, PPO uses a clipped surrogate objective to prevent excessively large policy updates, which commonly cause instability in environments with sparse rewards. The clip ratio was set to 0.2 throughout training, with a learning rate of 0.0003.

PPO operates on an Actor-Critic architecture. The Actor network outputs a probability distribution over discrete actions given the current observation. The Critic network estimates the expected cumulative reward for the current state. Both networks share a common feature extractor consisting of two fully-connected hidden layers of 256 units each with ReLU activations, processing the 7-dimensional observation vector to produce navigation decisions at each timestep.

## 3.2 Simulation Environment

The simulation was conducted using Microsoft AirSim (v1.8.1), an open-source high-fidelity robotics simulator built on Unreal Engine 4.27. AirSim provides realistic UAV physics, collision detection, and a Python API that enables programmatic vehicle control from external scripts. The urban scene was constructed using the AirSim Blocks environment, populated with cuboid obstacles of varying sizes to simulate urban buildings, as shown in Figure 1.

The UAV was configured as a SimpleFlight multirotor with API control enabled. The drone maintained a constant flight altitude of 3 metres above the origin throughout all tasks (Z = -3 in NED coordinates). The target point was positioned at coordinates (85.0, 110.0) metres in the horizontal plane, requiring approximately 138 metres of navigation with multi-directional movement.

## 3.3 Observation Space

At each timestep, the agent receives a 7-dimensional observation vector constructed from the UAV's current kinematic state:

- Dimensions 0-2: absolute position (x, y, z) in metres, obtained from AirSim's kinematics estimator.

- Dimensions 3-5: displacement vector to the target (dx, dy, dz) in metres, computed as target minus current position.

- Dimension 6: binary collision flag (1.0 if AirSim reports active collision, else 0.0).

This minimal observation design tests the agent's ability to learn effective navigation from limited sensory information, without reliance on depth cameras or lidar point clouds.

## 3.4 Action Space

The action space consists of four discrete cardinal movement directions: forward (+x), backward (-x), rightward (+y), and leftward (-y). Each action commands a constant horizontal velocity for a duration of 0.3 seconds via AirSim's moveByVelocityZAsync API, with flight altitude fixed at Z = -3 metres. The base cruise speed is 8.0 m/s, automatically reduced to 3.0 m/s when the drone is within 15 metres of the target to enable precise arrival.

## 3.5 Reward Function

The reward function was designed to encourage efficient, safe progress towards the target:

reward  = -0.1                             # per-step time penalty

reward += (prev_dist - dist) * 2           # dense progress reward

if collision:   reward -= 100              # collision penalty

if dist < 8.0:  reward += 200              # target arrival reward

The progress reward scales proportionally with the change in horizontal distance to the target, providing a dense learning signal that guides the agent continuously. The large collision penalty (-100) and arrival reward (+200) create a strong asymmetry that prioritises safe navigation over speed.

## 3.6 Curriculum Learning Strategy

A four-stage curriculum was applied to overcome the sparse-reward challenge inherent in long-horizon navigation tasks. Training began with a simple 20-metre forward navigation task and progressively increased in distance and directional complexity:

| Stage | Target Distance | Task Description | Approx. Timesteps |
| --- | --- | --- | --- |
| 1 | 20 m | Short-range forward navigation, minimal obstacles | 5,000 |
| 2 | 40 – 60 m | Medium-range straight navigation, moderate obstacles | 5,000 – 10,000 |
| 3 | 60 – 85 m + lateral | Introduction of Y-axis displacement, turning required | 10,000 |
| 4 | ~138 m (full task) | Complete path to (85, 110) m with multi-directional movement | 20,000+ |

At each stage, training continued from the previously saved model checkpoint using PPO.load(), allowing knowledge transfer across difficulty levels. This approach dramatically reduced training time and improved final performance compared to direct training on the full task.

# 4. Implementation

## 4.1 Software Stack

| Component | Tool / Library | Version |
| --- | --- | --- |
| Simulation Platform | Microsoft AirSim + Unreal Engine 4 | AirSim 1.8.1 / UE 4.27.2 |
| RL Algorithm | Stable-Baselines3 (PPO) | 2.4.1 |
| Gym Interface | Gymnasium | 1.0.0 |
| Deep Learning Backend | PyTorch | 2.4.1 |
| Python Environment | Anaconda (uav_rl env) | Python 3.8.20 |
| Version Control | GitHub | Victor7890-svg/drone-ppo-navigation |

## 4.2 Environment Implementation (drone_env.py)

The custom DroneEnv class implements the Gymnasium Env interface and wraps all AirSim communication. On initialisation, the environment establishes an API connection to the AirSim server and defines the 7D observation space and 4-action discrete action space. The reset() method resets the simulator, commands takeoff, and moves the drone to the target altitude before returning the initial observation. The step() method executes the selected velocity command, waits for completion, then queries the updated state and computes the reward.

A key implementation detail is the adaptive speed control: the drone flies at 8.0 m/s when more than 15 metres from the target, and reduces to 3.0 m/s within 15 metres. This prevents overshooting the 3-metre success radius, which would incur continued time penalties and delay episode termination.

## 4.3 Training Script (train.py)

The training script instantiates the DroneEnv environment and creates a PPO model using the MlpPolicy. For initial training, a new model is created from scratch. For subsequent curriculum stages, the existing model is loaded from disk using PPO.load() and fine-tuned with the updated environment parameters. Training progress is logged at each rollout interval, providing visibility into reward trends and stability metrics.

## 4.4 Evaluation Script (test_model.py)

The evaluation script loads the trained model and runs 20 deterministic test episodes with deterministic=True to ensure reproducible results. For each episode, it records whether the drone successfully reached the target (distance less than 3 metres) and whether any collision occurred. Success rate and collision rate are computed and reported at the end of evaluation.

# 5. Results and Analysis

## 5.1 Training Convergence (Stage 1)

The table below presents the complete training log from Stage 1 (20-metre navigation task, 5,000 total timesteps), demonstrating the fundamental learning dynamics of the PPO agent:

| Iteration | Timesteps | ep_rew_mean | ep_len_mean | approx_kl | Loss |
| --- | --- | --- | --- | --- | --- |
| 1 | 1,024 | -440 | 149 | — | 1,410 |
| 2 | 2,048 | -412 | 151 | 0.01018 | 987 |
| 3 | 3,072 | -234 | 130 | 0.01455 | 987 |
| 4 | 4,096 | -97.2 | 106 | 0.01177 | 706 |
| 5 | 5,120 | -12.0 | 86.1 | 0.01073 | 3,280 |

The mean episode reward improved from -440 at iteration 1 to -12.0 at iteration 5, a 97% improvement over 5,120 timesteps. Concurrently, mean episode length decreased from 149 to 86 steps, reflecting faster, more direct navigation. The approx_kl values (0.010 to 0.015) remained well within the recommended range throughout, confirming that PPO's clipping mechanism was functioning correctly and policy updates were stable. Entropy loss decreased gradually from -1.38 to -1.32, indicating progressive policy specialisation from exploration towards exploitation.

## 5.2 Final Evaluation Results

| Metric | Result |
| --- | --- |
| Total Test Episodes | 20 |
| Successful Navigations | 20 / 20 |
| Success Rate | 100% |
| Collision Rate | 0% |
| Navigation Distance | ~138 metres (multi-directional path) |
| Target Coordinates | (85.0, 110.0, -3.0) metres |
| Cruise Speed | 8.0 m/s |
| Approach Speed (within 15 m of target) | 3.0 m/s |
| Original Target Success Rate (from proposal) | > 90% (substantially exceeded) |

The trained model achieved a 100% success rate across all 20 evaluation episodes, with zero collisions recorded. This result substantially exceeds the 90% success rate specified in the project proposal. The consistent perfect performance across repeated trials confirms that the learned policy is robust and reliable, not the result of a single fortuitous run.

## 5.3 Key Findings

- Curriculum learning was essential: direct training on the full 138-metre task yielded near-zero reward improvement, as random exploration had negligible probability of reaching the distant target within the episode step limit.

- Dense reward shaping was critical: the distance-proportional progress reward provided continuous guidance that maintained learning momentum throughout all curriculum stages, even before any target arrivals occurred.

- Adaptive speed control improved precision: reducing velocity from 8.0 m/s to 3.0 m/s within 15 metres of the target was necessary to reliably achieve entry into the 3-metre success radius without overshooting.

- Minimal observation proved sufficient: despite containing no explicit obstacle proximity data, the 7D observation (position, direction-to-target, collision flag) provided enough signal for the agent to learn effective avoidance behaviour through trial and error.

# 6. Discussion

## 6.1 Advantages Over Traditional Methods

The PPO-based approach offers significant advantages over traditional path planning algorithms. A* requires a discretised grid map and re-planning when the environment changes, making it poorly suited to dynamic or partially observable settings. RRT produces probabilistically valid paths but is computationally expensive in dense environments and does not inherently optimise path smoothness or energy efficiency. The PPO agent, by contrast, produces navigation decisions in under 1 millisecond at inference time, with no runtime planning computation required.

The end-to-end learning approach also implicitly optimises for multiple objectives simultaneously — efficiency, safety, and speed — through the composite reward function, without requiring explicit multi-objective optimisation formulations.

## 6.2 Limitations and Future Work

- Observation design: the current 7D observation contains no direct perception of obstacle proximity. Incorporating depth sensor or lidar readings would enable proactive rather than purely reactive obstacle avoidance, likely improving performance in denser environments.

- Discrete action space: the four cardinal directions constrain navigational flexibility. A continuous action space with arbitrary velocity vectors would enable smoother, more energy-efficient trajectories and potentially higher success rates in complex scenes.

- Static environment only: the current scene contains only static obstacles. Extending the environment to include dynamic obstacles such as other drones would better reflect real urban airspace conditions.

- Sim-to-real gap: all results are obtained in simulation. Physical deployment would require domain randomisation during training and hardware-in-the-loop validation to bridge the reality gap.

- Training efficiency: at approximately 2 FPS simulation speed, curriculum training required several hours. GPU-accelerated or vectorised parallel environments would significantly reduce this overhead.

# 7. Conclusion

This project successfully developed and validated an autonomous UAV obstacle avoidance navigation system based on the PPO deep reinforcement learning algorithm, implemented in a realistic urban simulation environment using Microsoft AirSim and Unreal Engine 4.27.

The key contributions of this work are: (1) the design of a complete DRL navigation pipeline from scratch, including a custom Gymnasium environment, shaped reward function, and curriculum training infrastructure; (2) the application of four-stage curriculum learning to enable successful training on a long-horizon 138-metre multi-directional navigation task that would otherwise be intractable with standard flat training; and (3) the demonstration of 100% navigation success across 20 test episodes with zero collisions, substantially exceeding the original target of 90%.

These results confirm that PPO-based deep reinforcement learning is a viable and effective approach to autonomous UAV navigation in urban environments. The project establishes a solid foundation for future extensions including richer sensor integration, continuous action control, dynamic obstacle avoidance, and eventual real-world deployment.

# Appendix A: GitHub Repository

https://github.com/Victor7890-svg/drone-ppo-navigation

| File | Description |
| --- | --- |
| drone_env.py | Custom Gymnasium environment: AirSim connection, observation/action spaces, reward function, episode logic |
| train.py | PPO training script with curriculum learning support and model checkpointing via PPO.load() |
| test_model.py | Evaluation script: runs 20 deterministic test episodes and reports success and collision rates |
| README.md | Project overview, setup instructions, training strategy, and usage guide |

# Appendix B: AirSim Configuration (settings.json)

{

"SettingsVersion": 1.2,

"SimMode": "Multirotor",

"Vehicles": {

"Drone1": { "VehicleType": "SimpleFlight", "X": 0, "Y": 0, "Z": -2 }

},

"CameraDefaults": {

"CaptureSettings": [{ "ImageType": 0, "Width": 84, "Height": 84 }]

}

}